| ADVANCED AIRWAY MANAGEMENT Video Laryngoscopy-Assisted Endotracheal Intubation | PROTOCOL #: AW-001 LEVEL: PARAMEDIC Effective: [DATE] |
| --- | --- |

# INDICATIONS

- Respiratory failure/arrest unresponsive to BLS airway interventions

- Cardiac arrest

- GCS ≤ 8 with inability to protect airway

- Impending airway compromise (burns, anaphylaxis, trauma)

- Need for prolonged ventilatory support or airway protection

# CONTRAINDICATIONS

- Relative: Suspected cervical spine injury (maintain inline stabilization)

- Relative: Upper airway obstruction that cannot be visualized/bypassed

- Relative: Intact gag reflex without pharmacological adjuncts (RSI)

# EQUIPMENT REQUIRED

| Primary Airway Equipment | Adjuncts & Monitoring |
| --- | --- |
| □ IntuBlade video laryngoscope □ iPad/Android with IntuBlade app □ ET tubes (6.0, 7.0, 7.5, 8.0 mm) □ Stylet □ Bougie □ 10cc syringe for cuff inflation □ Backup DL blade (if needed) | □ Suction (rigid Yankauer) □ BVM with reservoir □ ETCO2 detector/waveform capnography □ SpO2 monitoring □ Commercial tube holder □ Supraglottic airway (backup) □ RSI medications (per protocol) |

# PROCEDURE

A. Pre-Intubation

- Ensure scene safety and adequate personnel (minimum 2 providers)

- Pre-oxygenate patient (100% O2 via NRB or BVM for 3+ minutes if time permits)

- Position patient optimally (sniffing position; ramped position for obese patients)

- Prepare all equipment; confirm suction is functional and within reach

- Connect IntuBlade to iPad/Android; confirm app is open and recording

- Establish IV/IO access

- If RSI indicated, administer medications per Pharmacologically-Assisted Intubation protocol

B. Intubation Procedure

- Insert IntuBlade into right side of mouth, sweeping tongue to the left

- Advance blade to vallecula (MAC technique); lift anteriorly to visualize epiglottis and cords

- Obtain epiglottoscopy view (visualize epiglottis with cords below — avoid going too deep)

- If secretions/blood present, suction using SALAD technique (continuous suction during laryngoscopy)

- Pass ET tube through vocal cords under direct/video visualization until cuff passes cords

- Remove stylet, inflate cuff with 5-10cc air

- Note tube depth at teeth (typically 21-23cm for adults)

C. Confirmation (MUST complete ALL)

- Waveform capnography (ETCO2) — REQUIRED for confirmation

- Auscultation of bilateral breath sounds and absence of epigastric sounds

- Visualization of chest rise

- SpO2 maintenance/improvement

- Secure tube with commercial holder; reassess after each patient movement

⚠ FAILED AIRWAY PROTOCOL If unable to intubate after 2 attempts (or SpO2 < 90%): 1. Return to BVM ventilation with OPA/NPA → 2. Insert supraglottic airway (i-gel, King LT, or LMA) → 3. If CICO: Perform surgical cricothyrotomy → 4. Contact Medical Control

Page 1 of 2

| ADVANCED AIRWAY MANAGEMENT Video Laryngoscopy-Assisted Endotracheal Intubation | PROTOCOL #: AW-001 LEVEL: PARAMEDIC Effective: [DATE] |
| --- | --- |

# INTUBLADE DEVICE OPERATION

Setup (< 30 seconds) - Open IntuBlade app on iPad/Android device - Remove IntuBlade from packaging - Connect IntuBlade to device (USB/Lightning connector) - Confirm video feed is active on screen - Position screen for optimal viewing (partner holds or mount) Post-Procedure - Video automatically uploads to cloud when connected to cellular/WiFi - Dispose of IntuBlade per biohazard protocols - No additional upload steps required by provider

# TROUBLESHOOTING

| Issue | Solution |
| --- | --- |
| No video on screen | Check connection; restart app; device still functions as DL with light |
| Poor visualization | Suction airway; reposition patient; ensure not inserted too deep |
| Blood/secretions obscuring | Use SALAD technique (continuous suction during laryngoscopy) |
| Difficulty passing tube | Use bougie; adjust stylet angle; consider smaller tube; BURP maneuver |
| Anterior airway | Optimize positioning (more head extension, ear-to-sternal-notch); use bougie |

# DOCUMENTATION REQUIREMENTS

Document the following in the ePCR:

| □ Indication for intubation □ Pre-oxygenation method/duration □ Medications administered (RSI) □ Device used (IntuBlade VL) □ Blade size | □ Number of attempts □ ET tube size and depth at teeth □ Confirmation methods (ETCO2 value) □ Complications (if any) □ Post-intubation vitals |
| --- | --- |

# QUALITY ASSURANCE

- All advanced airway cases are automatically flagged for QA review

- IntuBlade video automatically uploads and is reviewed by Medical Director

- Providers may be contacted for case review and coaching

- Video review is educational/non-punitive and supports continuous improvement

| Medical Director: ________________ | Review Date: ________________ | Page 2 of 2 |
| --- | --- | --- |